Technical Requirements

System Overview

The Wearable Visualizer POC consists of three primary components working together to enable real-time health monitoring from consumer wearables to clinical dashboards:

- Wearable Device (Samsung Galaxy Watch or Apple Watch) - Source of physiological data

- WebSocket Server - Intermediate data aggregation and forwarding service

- Streamlit Dashboard - Physician-facing visualization interface

Component Specifications

1. Wearable Device Layer

Supported Devices:

- Samsung Galaxy Watch series (Active 2, Watch 3, Watch 4, Watch 5, Watch 6)

- Apple Watch Series 4 and later (via HealthKit integration)

Data Collection:

- Heart Rate: Continuous sampling at 1-second intervals during "Workout Mode"

- Heart Rate Variability (HRV): RMSSD and SDNN metrics when available

- Blood Oxygen (SpO2): Periodic measurements (device-dependent)

- Skin Temperature: Where supported

- Activity Metrics: Steps, active minutes, exercise minutes

- Sleep Data: When available and permitted

Transmission Requirements:

- Bluetooth connection to paired smartphone

- Smartphone acts as data relay to wearable bridge service

- Data transmitted via HTTPS POST to local WebSocket server

- Fallback to local storage during connectivity interruptions

- Automatic resync upon connection restoration

2. WebSocket Server Component

Core Functionality:

- Real-time bidirectional communication channel

- Message queuing and buffering during client disconnects

- Data validation and normalization

- Basic anomaly detection for data quality

- Connection heartbeat and health monitoring

Technical Specifications:

- Language: Python 3.8+

- Framework: websockets library (v11.0+)

- Port: 8765 (configurable via environment variable)

- Message Format: JSON payload with timestamped sensor readings

- Security: TLS 1.2+ encryption (optional for POC, required for production)

- Scalability: Designed to handle 100+ concurrent connections

- Persistence: Optional SQLite buffering for offline resilience

API Endpoints:

- ws://[server]:8765/ws - Main WebSocket connection

- GET /health - Health check endpoint

- POST /data - Alternative HTTP ingestion endpoint

- GET /metrics - Prometheus-compatible metrics

3. Streamlit Dashboard Component

Visualization Features:

- Real-time heart rate chart with 2-second update interval

- Historical trends (1h, 6h, 24h, 7d views)

- Alert threshold configuration (tachycardia/bradycardia limits)

- Multi-patient overview grid (for clinical deployment)

- Data export capabilities (CSV, JSON)

- Alert history and acknowledgment tracking

Technical Specifications:

- Framework: Streamlit 1.31.0+

- Port: 8501 (configurable)

- Dependencies:

  - streamlit >= 1.31.0

  - pandas >= 2.2.0

  - plotly >= 5.15.0 (for interactive charts)

  - numpy >= 1.24.0

- Authentication: Basic HTTP auth (extendable to OAuth/OIDC)

- Deployment: Docker-ready configuration

- State Management: Streamlit session state + client-side caching

4. Data Flow Architecture

[Patient Wearable]

↓ (Bluetooth)

[Patient Smartphone]

↓ (HTTPS/WiFi or Cellular)

[Local Network Gateway]

↓ (WebSocket Protocol)

[WebSocket Server (Port 8765)]

↓ (WebSocket Protocol)

[Streamlit Dashboard (Port 8501)] ← [Physician Browser]

Data Pipeline:

- Wearable samples sensors continuously via Workout Mode

- Smartphone bridges data to local network via wearable bridge app

- Data sent to WebSocket server as JSON messages

- Server validates, timestamps, and forwards to connected dashboards

- Dashboard renders real-time updates with historical context

- All transactions logged for audit and billing compliance

5. Infrastructure Requirements

Development Environment:

- Python 3.8-3.11

- pip package manager

- Git version control

- IDE of choice (VS Code recommended)

Production Deployment Options:

- Option A: Single Raspberry Pi 4 (4GB+) as edge gateway

- Option B: Docker-compose deployment on existing hospital infrastructure

- Option C: Cloud deployment (AWS/Azure/GCP) with HIPAA-compliant configurations

- Option D: Hybrid edge-cloud with local buffering

Minimum Hardware Specifications:

- CPU: Quad-core 1.5GHz+ (Raspberry Pi 4 or equivalent)

- RAM: 2GB minimum, 4GB recommended

- Storage: 16GB SSD/eMMC (8GB for OS, 8GB for data buffering)

- Network: Ethernet + WiFi (dual-band preferred)

- Power: Reliable power supply with UPS capability for clinical settings

6. Security and Compliance Considerations

Data Protection:

- HIPAA-compliant data handling procedures

- End-to-end encryption for PHI in transit (TLS 1.2+)

- Role-based access control for dashboard users

- Audit logging of all data access and modifications

- Automatic logout after configurable inactivity period

Device Security:

- Secure pairing procedures for wearables

- Unique device identifiers for patient association

- Remote wipe capability for lost/stolen devices

- Regular security patching schedule for gateway devices

Regulatory Compliance Pathway:

- FDA Class II exemption for low-risk general wellness products

- HIPAA Business Associate Agreement (BAA) framework

- IEC 62304 medical device software lifecycle processes

- ISO 13485 quality management system alignment

7. Integration Points

EHR/Hospital Systems:

- HL7 FHIR API for patient demographics and encounter data

- SMART on FHIR compatible launch context

- CCDA export for longitudinal patient records

- HL7 v2.x interface engine compatibility (optional)

Clinical Workflow Tools:

- Secure messaging integration (e.g., TigerConnect, Vocera)

- Nurse call system alerts for critical events

- Scheduling system synchronization for routine checks

- Billing system interface for automated RPM code capture

8. Monitoring and Maintenance

System Health Metrics:

- WebSocket connection success rate

- Message latency (wearable to dashboard)

- Data completeness percentage

- System uptime and availability

- Resource utilization (CPU, memory, disk, network)

Maintenance Procedures:

- Automated backup of configuration and buffered data

- Log rotation and archival policies

- Security scanning and vulnerability assessment schedule

- Quarterly penetration testing for production deployments

- Annual HIPAA compliance reassessment

9. Scalability and Performance

Performance Benchmarks:

- Latency: <500ms from wearable sensor to dashboard display (95th percentile)

- Throughput: 1000+ messages/second per WebSocket server instance

- Concurrent Users: 50+ physicians simultaneously viewing dashboards

- Patient Capacity: 500+ monitored patients per server instance (with buffering)

Horizontal Scaling:

- Load balancing across multiple WebSocket server instances

- Redis-based message queuing for fan-out distribution

- Microservices architecture for component isolation

- Kubernetes orchestration for cloud deployments

Assumptions and Dependencies

Assumptions:

- Patients have compatible smartphones (Android 8.0+/iOS 12.0+)

- Patients consent to data sharing and monitoring

- Reliable internet connectivity available at patient locations

- Clinical staff have basic computer literacy

- Physician access to modern web browsers (Chrome, Firefox, Safari, Edge)

Dependencies:

- Samsung Health / Apple HealthKit platforms for sensor access

- Stable Bluetooth LE implementation in wearables

- HTTPS connectivity for smartphone-to-server communication

- Streamlit community support for visualization framework

- Open-source Python ecosystem for server components

Risk Mitigation

Technical Risks:

- Bluetooth Reliability: Mitigated by fallback to local storage and retry mechanisms

- Data Loss: Addressed through acknowledgment protocols and persistent buffering

- Platform Changes: Abstracted wearable interface layer for easier adaptation

- Browser Compatibility: Tested against latest versions of major browsers

Clinical Risks:

- Alert Fatigue: Configurable thresholds and alert suppression policies

- False Positives: Signal processing algorithms to reduce noise

- Clinical Overwhelm: Summary views and exception-based reporting

- Patient Non-adoption: Simplified patient experience with clear value proposition

Success Criteria

Technical Validation:

- 99% message delivery success rate under normal conditions

- <2-second end-to-end latency for critical alerts

- 95% data completeness over 24-hour monitoring periods

- Zero unplanned downtime during 30-day pilot period

Clinical Validation:

- 80% patient adherence to monitoring protocol

- Clinician-reported time savings in patient assessment

- Documentation of at least one clinical intervention enabled by early warning

- Positive patient satisfaction scores (>4/5 on Likert scale)

Business Validation:

- Successful billing of RPM CPT codes for pilot patients

- Positive ROI calculation within 6 months of deployment

- Scalability demonstrated to 10x pilot size without architectural changes